# INTRODUCTION

As with most plant used in underground mining operations, operators need to be aware of the hazards associated with operating a Jumbo drill rig.  Due to the design features and power sources of a large Jumbo electric drill, operators need to take extreme care when operating the machine or when others are working in the area.

Today we will talk about:

- making the drill site safe;

- hazards associated with operating the Jumbo;

- the operator’s responsibility in regard to themselves and other people;

- eliminating common operating errors and

- personal safety.

# safe drill site

Because of the nature of the work performed by a Jumbo drill rig, the area they are working in can be unsafe.  As the operator, you have an obligation to take all necessary steps to look after your own health and safety as well as that of your workmates.

?

“There are six safety checks that every underground worker should carry out.  How many can you name?”

Hand out sheet 1 – The five safety checks

The six checks to be carried out are:

- access:   make sure you can go to and from your work place safely;

- ventilation:   ensure the ventilation at your workplace is satisfactory;

- scaling – ground support:   before starting work, water down the face, backs and sides of the work area.  This will allow you to see any cracks in the rocks;

- equipment:   check your equipment and make sure you have all the correct tools.  Check that all tools and equipment are in a safe condition; and

- work procedures:   always check that you and your work mates are working in a safe manner and are following all site rules, procedures and regulations.  DO NOT TAKE CHANCES.

An easy way of making sure that you have a safe drill site is to plan your work in advance so that you can avoid accidents, damage and injuries.

Prior to starting the machinery or performing any work, you should walk around the Jumbo and around the work area. These inspections should be done before starting a new operation or after a break in an operation, especially after blasting has been carried out..

?

Hand out sheet 2 – Safe work site inspections

“What hazards will you be looking for during this inspection?”

During this inspection you would be looking for:

- insufficient scaling of the work area;

- debris laying around the work area;

- tools and machinery parts laying around;

- fumes from blasting operations;

- damaged or broken air, hydraulic and water pipes and fittings,

- slip, trip or fall hazards.

There are certain parts of the Mines Safety and Inspection Regulations that covers persons entering an underground work site.

?

“Can anyone name any regulations that may relate to entering the workplace ?”

Hand out sheet 3 - Regulations

- Fumes and Blasting 9.22.5  A person must not enter any area in a mine where blasting has taken place unless the person has checked that fumes resulting from the blast have been effectively dispersed and that the atmosphere is safe to breathe; and

- Wetting Down After Blasting 9.23  A person must not carry out any work in an underground mine following a blasting operation in that place, unless the backs, face, walls and broken rock in the place have been thoroughly wetted down and made safe.

Remember, most of the hazards around a Jumbo have the potential to be more serious than elsewhere and can cause permanent disability and fatalities.

## ELIMINATING COMMON OPERATING ERRORS

Although working on a Jumbo can be a high risk operation, following site procedures can eliminate a large number of these risks.

?

“Can anyone give me three examples of common operating errors?”

Hand out sheet 4 – Common operating errors

Common operating errors include:

- coupling and uncoupling incorrectly – this can be an extremely dangerous procedure if not carried out correctly;

- incorrectly isolating when parking the machine – fires can occur if both the AC and DC systems are not isolated correctly;

- not assessing the ground conditions – some ground conditions may look okay when seen from one view point but when seen from another angle, may be seen as being hazardous;

- boring into butts – this is a dismissable offence at some sites; and

- boring within six metres of a misfire – follow all site rules and regulations when dealing with misfires.

A pre-requisite to any job is to ensure that you follow all site rules and procedures.  Remember, most procedures have been written so that you can work safely around an existing hazard.  By not following these site rules and procedures, you may cause injury or harm to yourself, your workmates or machinery.

?

Hand out sheet 5 – Injuries and their causes

“What are some examples of injuries, and their causes, that may result from not following rules or procedures?”

- Some of the injuries that may occur, and their causes, are:

- cuts and abrasions, serious injury or death caused by rock falls due to inadequate ground inspection/support/scalilng.

- electrocution caused by the incorrect isolation of the power supply; and

- fuming caused by entering a work site after blasting without checking the atmosphere.

- Many accidents have been caused by the release of pressurised material such as, compressed air or pressurised water.  If not treated with respect, these materials, even under relatively low pressures, can cause serious injuries.

- It is worth noting that if pressurised material is not being used, the pressure will slowly build up and the initial burst when the equipment is turned on can be significantly higher that the normal rating for the equipment.

?

“What types of injuries can be caused by compressed air and pressurised water?”

Hand out sheet 6 – Pressurised material injuries

## Injuries that can be caused by pressurised materials include:

- fractures, bruises and abrasions – hoses under pressure will flail around with enough force to break bones;

- eye damage – if directed at the eyes they can cause serious injury, even at low pressures;

- ear damage – as with eye damage, even low pressures can cause serious injury; and

- embolism – where compressed air can enter through the skin and cause air bubbles in the blood system.

- An air embolism can be caused by 4psi of pressure.

- LEAVING THE MACHINE UNATTENDED

- When leaving the Jumbo unattended you must switch the power off and isolate both the AC and DC systems.  Fires have occurred in the past because operators have not followed the site rules and regulations.

- The Mines Safety and Inspection Regulations state the following in reference to Unattended Trackless Units.

- Unattended Trackless Units  (10.41.1)  The operator of a trackless unit in an underground mine must ensure that the unit is not parked or left unattended unless:

- the engine or power supply has been switched off; and

- the park brake has been applied.

## OPERATOR RESPONSIBILITY

Operators are responsible for themselves, other people working around the Jumbo and to the equipment they are working with.  Safety legislation calls for the employer to ensure the workplace is free of hazards.  The legislation also places responsibility on the employee to work safely, follow the directions of the employer and use the supplied PPE.

?

“What steps can you take to make people aware of the hazards around a Jumbo drill rig?”

Hand out sheet 7 – Making people aware of hazards around a Jumbo drill rig

Steps you can take to make people aware of hazards are:

- place signs at points of entry to the work site;

- ensure that no-one goes forward of the operator’s cab whilst the Jumbo is operating;

- barricade dangerous areas;

- ensure any guarding or protective barriers are in place and in good order; and

- practice good housekeeping and keep the area clean.

The operator also has other responsibilities to carry out when operating the Jumbo.  These may involve site rules or regulations.

“Can you give some examples of the operator’s other responsibilities?”

?

Hand out sheet 8 – Operator’s responsibilities

Some of the further responsibilities of the operator are:

- to carry out pre-start checks;

- to conduct the five point safety checks;

- to follow all operating procedures;

- to report all hazards that cannot be fixed;

- to report all injuries or harm that may occur;

- to keep your work area clean and tidy;

- to check for Danger or Out of Service tags;

- to use correct manual handling procedures for heavy equipment; and

- to give an accurate report as to the condition of your workplace to your cross mate and Supervisor.

# PERSONAL SAFETY

You need to think seriously about your personal safety because, ultimately, you are responsible for your own safety.  All operations have rules, regulations and procedures to guide you in your actions.  But, over and above these, a little common sense can save you from serious injury.

Long hair and loose clothing can cause serious accidents as they have a potential to get caught on, or in, moving or rotating parts.  If you have long hair, you must tie it back or wear a hairnet or hat.  You should also wear tight fitting clothing and make sure you have no flapping sleeves or shirt tails.

Personal Protective Equipment is mandatory on mining sites.  However, because of the nature of the drilling, it is even more important when working on Jumbo’s.

?

- “What are the basic Personal Protective Equipment requirements?”

Hand out sheet 9 – PPE requirements

The Personal Protective Equipment requirements are:

- hearing protection;

- eye protection;

- steel cap boots;

- safety helmet;

- hand protection;

- means to constrain long hair;

- tight fitting clothing, long trousers and sleeved shirt or sleeved overalls;

- a self-rescuer; and

- a cap lamp battery attached to your safety belt.

## SUMMARY

Working with, or around, Jumbo drill rigs can be a hazardous operation.  To work safely you need to be aware of the hazards that are, or could be, present. To be aware, you should do a hazard analysis for each part of the operation. The pre-start analysis should be a formal walk around.  When performing the job, you should try to get into the habit of mentally assessing each task as you are working.

You should eliminate the short cuts that are usually taken by some operators.

Ensure that no-one goes forward of the Jumbo whilst the machine is operational.

If you have to climb in areas of rotating machinery, make sure the machine is turned off.  Do not leave controls unattended if they present any danger to you or any other person.

Ensure you only use the correct tools for the job and always maintain them in good condition.  Make sure the controls on the Jumbo are correctly labelled so that, in an emergency, any person can turn the machine off.

When doing maintenance or repairs, make sure the work area and the machine are safe to work on.  Isolate the equipment to be worked on and make sure that it cannot be operated by mistake.

Your wellbeing on the job is your responsibility and you can do your part simply by using a bit of common sense.

Make sure you do not have long hair or loose clothing flapping about.

Think about the consequences of taking a short cut.

Keep the area clean and tidy.

Further References for the Supervisor

Mines Safety and Inspection Act Regulation 1995

Department of Minerals and Energy WA Safety Bulletin No 31

WorkSafe Western Australia

DME - Significant Incident Reports 17,98 and 8

Risk Taking Behaviour in the western Australian Underground Mining Sector

Fatalities Taskforce into the Western Australian Mining Industry

Notes

Notes

Notes