# INTRODUCTION

Many fatal crashes are caused by driver fatigue.  The extent that fatigue contributes to traffic accidents is higher than statistics indicate. Fatigue will seriously impair your driving ability long before you become drowsy.

Some of you work in remote areas and need to travel long distances to get to work.  It is vitally important that you recognise the signs of fatigue and take the necessary precautions to reduce the chance of having a vehicle accident.

It is your responsibility to stop and rest when you realise that you are becoming fatigued.

To minimise the likelihood of driver and passenger injuries and fatalities, we will be discussing:

- the symptoms of fatigue;

- the causes of driver fatigue;

- how fatigue affects driving;

- how to prevent fatigue; and

- what to do when you are fatigued.

# THE SIGNS of FATIGUE

Once fatigue sets in, the only way to improve your concentration is to sleep.

?

“What are the signs of fatigue setting in”?

Hand out sheet 1 - The signs of fatigue

The signs of fatigue are:

- feeling cramped or fidgety;

- your vehicle wandering on the road;

- your speed varying;

- overtaking vehicles startling you;

- yawning constantly;

- your mind is wandering;

- your eyelids are heavy; and

- you are impatient.

An alert and healthy driver is aware of the vehicle’s performance, reacts to a given situation with the correct actions and feels little or no stress.

Colds, flu, and headaches can impair your ability to concentrate.  Your reactions will be slower and you may misjudge situations.

Falling asleep at the wheel causes many deaths and serious accidents.  Long before a driver falls asleep, fatigue can seriously impair his or her driving ability.

Weariness is more dangerous when driving in your normal sleep time.  Fatigue is frequently felt at the onset of darkness.  Visibility is reduced and there is a higher risk of accidents, even for an alert driver.

# THE DANGEROUS HOURS

Nearly twice as many people are killed at dusk than during the daylight hours.

Hand out sheet 2 – The dangerous hours

Noon

Midnight

Night

Day

Dawn

Dusk

# AVOIDING FATIGUE

The first step towards avoiding fatigue, is to plan your trip.

There are a number of steps and precautions that you should take prior to travelling.

Your vehicle, whether it is a car, truck or van, must be in sound mechanical condition with all safety and support equipment operating correctly.

#### Pre-start checks

By performing standard vehicle pre-start checks, you will identify any deficiencies.

Hand out sheet 3 – Pre-start checks

Typical things to check include:

- brakes;

- steering;

- lights ( headlights, oil, brake, indicators, clearance);

- tyres for tread depth and damage (including spares);

- correct tyre inflation;

- oil and water levels;

- clean windscreen;

- windscreen washers are working;

- seat and steering adjustment;

- mirrors adjusted to suit the driver; and

- seat belts.

If you are towing a trailer there are additional requirements.

?

“What are these”?

Hand out sheet 4 – Additional trailer pre-start checks

- Semi Trailer

Additional checks include:

- brake air line connections;

- security of tool and chain boxes;

- appropriate safety signs are available and in place; and

- breakdown equipment is in good order.

- Light vehicle trailers, caravans, car trailers, welders and air compressors.

Additional checks include:

- ensure mirrors provide clear vision down both sides of the towed equipment;

- safety chain is short enough to stop the hitch from hitting the ground if the trailer disconnects;

- load capacity is suitable for the trailer equipment;

- distribution of load to ensure the hitch is not overloaded;

- trailer registration; and

- coupling or hitch locked.

## Travel Plan

Now you have established that your vehicle and equipment are in a sound operating condition, you must plan the trip in such a way as to reduce the onset of fatigue.

?

“What are the basic things you need to build into your plan?”

Hand out sheet 5 – Travel plan

The basics of your plan should be:

- where possible, plan your trip to avoid long drives after a full day’s work;

- plan your work to avoid long drives before you start work;

- have a light meal before you leave;

- take advantage of the free coffee offered by most road houses;

- avoid drinking alcohol before or during the trip; and

- if you have a passenger, share the driving and change over every hour.

As we discussed earlier, the onset of fatigue is greater at dusk and nearly twice as many people are killed during this part of the day.  Plan your trip to avoid travelling at dusk.

# NIGHT DRIVING

Driving at night requires a far higher degree of awareness.

?

“What are the additional precautions needed for night driving?”

Hand out sheet 6 – Night driving

Night driving precautions include:

- allowing more distance between you and the next vehicle (you need more stopping room);

- driving slower (darkness makes perception slower and 90% of driving decisions are based on what you see);

- keeping windscreens, mirrors and lights clean and not allowing your windscreen to fog up; and

- loading trailers and vehicles so they do not affect the position of your lights.

Note: Overloading vehicles and trailers can raise the position of your headlights to an extent that, even on low beam, your lights can blind oncoming traffic.

If it is not possible to balance the vehicle, use the following procedure to adjust your lights:

Hand out sheet 7 – Headlight adjustment

- Face your vehicle one metre away from a wall.

- Put a mark on the ground and a corresponding mark on one of the tyres.

- Find the headlight adjusting screws under the bonnet.

- Mark the position of the screws.

- Turn the lights on and set them to low beam.

- Mark the concentration of the light beam on the wall.

- Repeat step 6 for high beam.

- Load you vehicle.

- Re-position the vehicle on the one metre mark.

- Turn on the lights and set them to low beam.

- Locate the concentration of light at the center. Adjust the lights to the mark you made before loading the vehicle or trailer.  This adjustment should be sufficient to cover both high and low beam.

- Readjust your lights to their original setting when you have unloaded your vehicle.

Hand out sheet 8 – Additional skills for night driving

The additional skills required for night driving include keeping a lookout for signs of oncoming traffic, such as light flashes on corners and a glow on the approach to a hill.

- Be prepared for approaching vehicles that may have their lights on high beam Look to the side of the road, not directly at the lights.

- Slow down when you dip your lights as this reduces your visibility.

Note:  IT ALSO AFFECTS YOUR BREAKING DISTANCE.  At approximately 110 km/h you are at the limits of your lights.

## Falling Asleep at the Wheel

Country driving is particularly hazardous to a fatigued driver. A road that is straight and monotonous can cause you to fall asleep because your brain has no stimulation.  Then again, a winding road with avenues of trees can be just as mesmerising to a fatigued driver.  The sunlight filtering through the trees can have a hypnotic affect on you and cause you to fall asleep.

?

“What are some ways to avoid falling asleep at the wheel?”

Hand out sheet 9 – How to avoid falling asleep at the wheel

Some ways of remaining refreshed for long drives are:

- get plenty of sleep before setting out;

- stop every two hours for a drink or take a short walk;

- when you feel drowsy, STOP immediately and have a nap;

- if you have a passenger, alternate the driving every hour;

- open the window, fresh air will clear your head;

- play music or the radio to stimulate your brain; and

- do not drink alcohol or take any medication prior to, or during, your trip.

# SUMMARY

Driving excessive hours without sleep, having a cold, flu or headache or setting out on a long drive after a full day at work will contribute to driver fatigue.

Driving at night requires a far higher degree of awareness.  Nearly twice as many people are killed at dusk than during daylight hours.

The signs of fatigue are easily identified.  If you find your self fidgeting, yawning unavoidably, varying your speed, getting impatient or day dreaming - STOP.  You will be surprised how even a twenty minute nap will restore your energy and alertness.  If need be, stop and nap every few hours.

Falling asleep at the wheel can be fatal.  The only way to avoid this is to SLEEP WHEN YOU NEED IT.

Further References for the Supervisor/Presenter

Survival Motorists Emergency Handbook

Published by: Survival Emergency Products

WorkSafe and WorkCover

General Publications

Notes

Notes

Notes

Notes