# INTRODUCTION

A serious injury resulting from a light vehicle accident can change your life.  A serious accident can take your life.

The safe operation of light vehicles depends on you understanding two key aspects:

- prevention; and

- response.

Light vehicle driving, to most of us, becomes automatic.  How many times have you acted or reacted without thinking when driving?  This is called “automatic response”.

Investigation of many light vehicle accidents shows that one of the most common causes of accidents is “not driving to suit the conditions”.

There are many factors that contribute to this.  To develop a common understanding of the factors contributing to vehicle accidents, today we will be discussing:

- knowing your vehicle;

- correct vehicle pre-use inspections;

- common driving hazards as potential causes of accidents;

- driving skills;

- vehicle control; and

- driving to suit the conditions.

There are many causes of accidents with cars, utilities and four wheel drive vehicles. The starting point in safe driving is to know your vehicle, recognise the hazards and adjust your driving to suit.

# Knowing your vehicle

Every vehicle is different and has different handling characteristics.  Because we drive a variety of vehicles (hire cars, company cars etc.) we must establish how each will perform, or react, before we set out.

?

“What are some of the things we should look for in order to identify the difference between vehicles?”

Hand out sheet 1 – Identifying handling characteristics

The things to look for are:

- steering (is it heaver or lighter);

- the position of the driver's seat (can you set it to your normal driving position);

- mirror adjustments (can you see both sides of the vehicle);

- clearances around the vehicle (wider, narrower, shorter longer); and

- brakes (softer, harder, less  force to apply or more).

All these factors have an effect on our ability to control the vehicle in changing conditions.

# Vehicle pre-use inspections

Before using any vehicle, you should conduct a pre-start or pre-use inspection.

?

“What are some of the checks you would conduct during a pre-use inspection?”

Hand out sheet 2 – Pre-use inspections

A typical pre-use check would include:

- the brakes;

- the steering;

- the lights (head, tail, brake and indicators).

- the tyres for tread depth and damage (including spares);

- the correct tyre inflation;

- oil and water levels;

- clean windscreen;

- windscreen washers;

- seat and steering wheel adjustments;

- the mirrors are adjusted to suit the drivers; and

- that seat belts are installed and in good condition.

If you are towing a trailer there are additional requirements.

“What would the additional inspection requirements be if you were planning to tow a trailer?”

?

Hand out sheet 3 – Trailer pre-use inspections

- The following additional checks must be included if you are towing light vehicle trailers, caravans, car trailers, welders or air compressors:

- ensure the mirrors provide clear vision along both sides of the equipment being towed;

- make sure that the safety chain is short enough to stop the hitch from hitting the ground if the trailer disconnects;

- the lights (trailer, side, clearance, brake etc.);

- the load capacity is not exceeded;

- planned distribution of load to ensure the hitch is not overloaded;

- the tyres for tread depth and any damage (including spares);

- the trailer registration; and

- make sure the coupling or hitch is locked.

Now that we understand the characteristics and physical state of the vehicle, we need to consider the potential hazards.

# common Driving hazards

Every day we are faced with different driving conditions.  The hazards can vary during a single trip.

“What are some of the hazards we could be faced with during our next trip?”

?

Hand out sheet 4 – Driving hazards

The hazards we may be faced with include:

- unsafe road conditions (slippery, loose gravel on the edge of roads, mud, oil, corrugations);

- bad weather (rain, fog, sun glare, high winds);

- livestock (domestic, farm and native animals);

- mechanical failures (breakdowns, tyre blow outs, loads falling from vehicles); and

- pedestrians.

We can learn to anticipate and manage these hazards.

# Driving Skills

Everyone passes a driving test to obtain a licence.  Just because we have a licence does not make us a good or safe driver.

It is the adoption and development of skills and the ability to anticipate hazards or adverse conditions that makes us a safer driver.

?

“What are some of the skills we need to develop and use in order to become a safer driver?”

Hand out sheet 5 – Driving skills

Some of the basic skills we need to develop are:

- looking ahead and checking our mirrors regularly;

- driving within the speed limit;

- anticipating other drivers actions (watch break lights, watch the position of the vehicle on the road);

- complying to signs and road markings;

- keeping both hands on the wheel;

- keeping a safe distance from the vehicle ahead;

- driving at a consistent speed;

- indicating well in advance of your planned turn;

- only overtaking when we have clear visibility;

- driving in the left lane, unless overtaking; and

- not assuming anything - being courteous and patient.

Along with the skills discussed here, there are a number of bad driving habits we may have developed and should eliminate.

?

“What are some of our bad driving habits that need to be eliminated?”

Hand out sheet 6 – Bad driving habits

Some possible bad driving habits include:

- using a hand held mobile phone;

- eating or drinking;

- being under the influence of  alcohol and drugs;

- turning without indicating;

- playing loud music, which drowns out the sound of emergency sirens and close vehicles;

- driving while fatigued;

- braking suddenly and lane changing; and

- showing a lack of patience and courtesy.

While the development of basic driving skills and the elimination of our bad driving habits will reduce the risk of accidents, we need the more advanced skill of how to react in order to keep control of our vehicle.

# vehicle control

?

“What does vehicle control mean to you?”

Hand out sheet 7 – Vehicle control

Vehicle control means that you are always in control. You can anticipate hazards and therefore avoid the need for sudden manoeuvres that could cause you to skid.

To increase your chances of maintaining control of your vehicle, you should:

- avoid the edges of the road;

- avoid sudden braking;

- drive well within your lane;

- keep well left on the crest of hills and sharp corners;

- avoid “u” turns;

- accelerate slowly on wet roads;

- drive slower on gravel roads and slow down on corners; and

- lower your headlight beam to avoid blinding oncoming drivers.

# Driving to the conditions

A large number of accidents are caused by not driving to the road conditions.

?

“What does 'driving to the conditions' mean?”

Put simply, we need to adjust our driving to compensate for:

- the vehicle we are using;

- the potential hazards; and

- our skill and experience.

It’s our ability to recognise road conditions and adjust our driving accordingly, that will reduce the risk of us having an accident.

## SUMMARY

The safe use of light vehicles depends on how well we, and the vehicle, are prepared, our driving skill, our ability to recognise hazards and how well we manage emergency situations

Today we have only looked at the basics of understanding your vehicle. You need to think about how you and your vehicle will react in a given situation.

The need for regular inspections and pre-use checks is critical to the safe operation of your vehicle.

The development of driving skills, and not driving beyond your skills, is the other key factor in safe operation.

Finally, the ability to anticipate will greatly reduce the risk of having a vehicle accident.

Further References for the Supervisor/Presenter

Road Safety Council of Australia

Notes

Notes

Notes

Notes