# INTRODUCTION

Barricading is protective equipment used to prevent people being unnecessarily exposed to a hazard.  Barricading, if properly used, achieves this by:

- bringing the presence of a hazard to peoples attention; and

- restricting access to an area of the workplace.

- Like other types of protective equipment, if barricading is not used properly, it is ineffective in preventing exposure to hazards.

- Barricading can be either permanent or temporary.  Permanent barricading, such as fences and screens, is used to isolate and prevent access to large parts of fixed machinery and equipment which:

- are potentially hazardous; and

- cannot be fully encased or guarded.

- Temporary barricading is used to prevent access to an area where:

- a temporary hazard exists; or

- when certain work creates a hazard for other people nearby.

- Today, we will be discussing temporary barricading and specifically:

- the types of barricading;

- when they should be used; and

- how they should be used correctly.

# WHEN BARRICADING SHOULD BE USED

?

“What are some examples of when barricading should be used?”

Hand out sheet 1 – When barricading should be used

- Barricading should be used when carrying out:

- hot work at heights, such as welding and cutting, where sparks and hot slag could drop down onto people below;

- excavation work where people and machinery could move too close to the edges of the excavation; work at heights with Elevating Work Platforms;

- crane lifting operations where people could walk into the area where loads are being slewed, raised or lowered;

- hazardous material removal, such as asbestos, to prevent people with inadequate Personal Protective Equipment from entering the area;

- floor repairs to prevent slip and trip accidents from occurring;

- work in manholes; and

- abrasive sandblasting work.

- This does not include all of the possible situations where temporary barricading should be used.  As a general rule, temporary barricading should be used whenever:

- there is a potential hazard which people may not be aware of; and

- where they could be injured if allowed to enter the area without following certain safety precautions.

# Types of Barricade Tape

- The most commonly used temporary barricading on construction sites is barricade tape. The two main types used are:

- red and white tape; and

- black and yellow tape.

- Both types of barricade tape have a specific meaning and use.

?

“When should red and white barricade tape be used?”

Hand out sheet 2 – Use of red and white tape

- Red and white barricade tape means “Danger”.

- This tape should be used to barricade areas where a high risk hazard exists and there is a definite risk of injury or harm if certain safety precautions are not followed.

- Some examples of when red and white barricading tape should be used include:

- missing floor or grid mesh sections;

- unstable buildings and structures; and

- below areas where hot work is being carried out.

- You should only enter an area which is barricaded with red and white tape if you are:

- authorised by your Supervisor;

- fully aware of the nature of the hazard; and

- wearing the appropriate Personal Protective Equipment.

?

“When should black and yellow barricade tape be used?”

Hand out sheet 3 - Use of black and yellow tape

- Black and yellow barricade tape means “Caution - Risk of Danger”.

- This tape should be used to warn people of a potential hazard.

- Some examples of when black and yellow barricading tape should be used include:

- around the area where a crane lift is being carried out;

- underneath leaking elevated process pipes;

- around the slewing radius of an Elevating Work Platform while it is operated in a public place or thoroughfare; and

- around the perimeter of minor excavations.

- You should identify the specific nature of the hazard and take the appropriate safety precautions before entering an area which is cordoned off by black and yellow tape.

# Barricade Webbing

Hand out sheet 4 – Barricade webbing

- An alternative method of barricading is barricade webbing which is made from:

- rigid plastic sheeting; or

- tear-proof fabric.

- This provides a better visual barrier than tape and is often used for barricading large excavations.

# Post and Rail Barricading

Hand out sheet 5 – Post and Rail barricading

- Barricade tape and webbing only highlight an area where a hazard exists and do not provide a strong physical barrier.

- When barricading needs to be higher than 2 metres or provide a physical barrier, barricade tape and webbing should not be used.

- A barricade made from steel posts and rails should be instead.  This type of barricading is used on the perimeters of scaffold platforms, or fixed platforms for fall protection.  A mid-rail or steel mesh in-fill is generally used to complete the barricade.

# Using Barricading

?

“What are some of the rules you should follow when erecting barricades?”

Hand out sheet 6 – Using barricading

- Position the barricading so that it provides adequate warning and protection.

- Ensure the perimeter of the barricading extends out far enough to protect against contact with the hazard.

- Position the barricading at a height so it is clearly visible.

- Tie off barricading tape to posts so it is kept in position.

- Posts should either be driven into the ground or fitted with a heavy base to prevent the barricading from falling down.

- Position safety signs at different points around the barricade where they can be easily seen. People need to know why the area has been barricaded so they can take the proper precautions before they enter the area.  Barricading, on its own, does not provide adequate information.

- You should use:

- Danger warning signs with red and white tape; and

- Caution signs with black and yellow tape.

- During hours of darkness position flashing warning lights around barricaded areas such as excavations.

- Temporary barricading should be dismantled as soon as the hazard no longer exists.  You should only dismantle barricading if:

- the hazard has been removed; and

- you have been authorised by your Supervisor.

- Temporary barricading should only be a temporary measure to protect people against a hazard.  It should only be used to provide protection until the hazard has been removed.

# Summary

- When using barricading you should:

- select the correct type of barricading for the type of hazard;

- correctly erect the barricading to provide maximum warning and protection;

- use safety signs with barricading to inform people of the hazard nature;

- only enter barricaded areas if you have authorisation from your Supervisor;

- check you are wearing the correct Personal Protective Equipment before you enter a barricaded area; and

- remove barricading when the hazard no longer exists.

Notes

Notes

Notes

Notes

Notes